Key 10 Points of Hydraulic Power Unit (HPU) Design

1. Basic Parameter Determination

- Clarify the working pressure, flow rate, and driving power required by the system to avoid insufficient power or energy waste.

- Determine the working medium (hydraulic oil type, viscosity grade) according to ambient temperature, load characteristics, and service life.

- Define the cycle mode: continuous operation, intermittent operation, or high-frequency start-stop, to match the heat dissipation and component durability design.

2. Oil Tank

- Reasonably calculate the effective volume of the oil tank, generally 3 to 5 times the system flow rate to ensure oil heat dissipation and bubble separation.

- Set up oil suction and return partitions, liquid level gauges, oil temperature gauges, air breathers, and drain valves.

- Use anti-rust and anti-corrosion materials, with smooth inner walls to reduce oil residue and pollution.

3. Pump

- Select the appropriate hydraulic pump: gear pump (low pressure, low cost), vane pump (medium pressure, low noise), piston pump (high pressure, high efficiency).

- Match the driving motor with rated power, speed, insulation grade, and protection level (IP54/IP65) based on working conditions.

- Reserve installation space for shock absorbers to reduce vibration and noise transmission.

4. Valve & Circuit Design

- Configure control valves (directional, pressure, flow control valves) reasonably to meet speed, pressure, and direction regulation requirements.

- Optimize the pipeline layout to reduce bending, avoid eddy current and pressure loss, and use high-pressure hoses and hard pipes matching the pressure level.

- Add overload protection, pressure relief circuits, and check valves to prevent hydraulic shock and oil backflow.

5. Cooling & Heating System

- Design an air cooler or water cooler according to the calorific value to control the oil temperature within 30–55°C.

- Equip an electric heater for low-temperature environments to ensure the viscosity of hydraulic oil is within the usable range.

- Install temperature sensors and automatic temperature control switches.

6. Filtration & Pollution Control

- Install oil suction filters, pressure filters, and return filters to control oil contamination level (generally NAS 8–10).

- Set up filter clogging alarm devices to remind maintenance and replacement.

- Ensure the sealing performance of the entire system to prevent dust and moisture intrusion.

7. Noise & Vibration Control

- Adopt low-noise hydraulic pumps and motors, and install shock-absorbing pads and mufflers.

- Optimize pipeline fixation to avoid resonance and reduce fluid noise caused by throttling and impact.

8. Safety & Reliability Design

- Set up emergency stop buttons, pressure relief valves, and explosion-proof devices for high-risk working conditions.

- Adopt redundant design for key components to improve system stability.

- Reserve test ports for pressure, flow, and oil quality detection to facilitate maintenance.

9. Installation & Maintenance Convenience

- Arrange components in a compact and reasonable layout for easy observation, disassembly, and replacement.

- Reserve enough space for pipeline connection, filter replacement, and oil filling/draining.

- Match the installation size with the equipment base to ensure stability.

10. Environmental & Application Adaptability

- Strengthen protection for outdoor, high-temperature, low-temperature, humid, or corrosive environments.

- Comply with industry standards and safety regulations for special applications such as marine, mining, and mechanical equipment.